Demonstration of Network Communication Models (OSI / TCP-IP) Using a Network Simulation Tool

To demonstrate and analyze the OSI and TCP/IP network communication models using a network simulation tool (e.g., Cisco Packet Tracer or Wireshark).

## Objective

- Build a local area network (LAN) topology in a simulation environment.

- Map operational protocols to their respective OSI and TCP/IP model layers.

- Trace packet data encapsulation and decapsulation during node-to-node communication.

- Inspect packet headers and protocol behaviors using simulation tools.

## Network Setup & Parameters

### Simulation Environment

- Tool: Cisco Packet Tracer / Wireshark Analyzer

- Topology: Star Topology (2 End Devices, 1 Switch, 1 Router, 1 Web Server)

### Network Configuration Table

| Device Name | IP Address | Subnet Mask | Default Gateway | Role |
| --- | --- | --- | --- | --- |
| PC-A | 192.168.1.10 | 255.255.255.0 | 192.168.1.1 | Client PC |
| Switch-1 | Unmanaged / Layer 2 | N/A | N/A | Local Traffic Interconnect |
| Router-1 | 192.168.1.1 | 255.255.255.0 | N/A | Default Gateway Router |
| Web Server | 10.0.0.50 | 255.0.0.0 | 10.0.0.1 | HTTP Web Host |

## Mapping Network Communication Models

| OSI Layer No. | OSI Layer Name | TCP/IP Layer Equivalent | Protocols Sampled | Data Unit (PDU) |
| --- | --- | --- | --- | --- |
| Layer 7, 6, 5 | Application, Presentation, Session | Application | HTTP, HTTPS, DNS, FTP | Data / Payload |
| Layer 4 | Transport | Transport | TCP, UDP | Segment |
| Layer 3 | Network | Internet | IP, ICMP, ARP | Packet |
| Layer 2 | Data Link | Network Access | Ethernet, MAC | Frame |
| Layer 1 | Physical | Network Access | RJ-45, Fiber, Signals | Bits |

## Simulation Execution & Data Flow Analysis

### Packet Flow Scenario

Client PC (192.168.1.10) sends an HTTP Request to Web Server (10.0.0.50).

### Step-by-Step Encapsulation Process

- Application Layer: Web browser generates HTTP GET request data.

- Transport Layer: TCP appends Source Port (50122) and Destination Port (80), formatting a TCP Segment.

- Network Layer: IP protocol appends Source IP (192.168.1.10) and Destination IP (10.0.0.50), forming an IP Packet.

- Data Link Layer: Ethernet appends MAC addresses (Source MAC of PC, Destination MAC of Router Gateway), forming a Frame.

- Physical Layer: Frame bits are converted into electrical signals and passed across the physical cable.

[ Data ] → [ TCP | Data ] → [ IP | TCP | Data ] → [ Eth | IP | TCP | Data | FCS ] → Binary Bits

## Experimental Verification Table

| Event Step | Protocol | Outgoing Encapsulation Details | Layer Tested | Observation Result |
| --- | --- | --- | --- | --- |
| 1. DNS Query | UDP / DNS | Query www.example.com to IP | Application | IP Resolved Successfully |
| 2. ARP Request | ARP | Broadcast MAC discovery for Gateway | Data Link | Gateway MAC captured |
| 3. TCP Handshake | TCP | SYN → SYN-ACK → ACK packets | Transport | Connection Established |
| 4. HTTP Request | HTTP | HTTP GET / Index.html | Application | Data Transferred |

## Results

- Successfully constructed a simulated network and observed communication across all layers.

- Captured and inspected protocol data units (PDUs) at each stage of network traffic flow.

- Validated the encapsulation and decapsulation processes during cross-network routing.

## Conclusion

Network simulation tools offer clear visibility into data transmission mechanics. By tracking packet headers through OSI and TCP/IP layers, we observe how higher-level application requests are translated into routable network packets and physical signals.